1.0	STATUTORY AUTHORIZATION, FINDING OF FACT, STATEMENT OF 	PURPOSE, TITLE AND GENERAL PROVISIONS

1.1	STATUTORY AUTHORIZATION

This ordinance is adopted pursuant to the authorization in ss. 61.35 and 62.23, for villages and cities; 59.69, 59.692, and 59.694 for counties; and the requirements in       s.87.30, Stats.

1.2	FINDING OF FACT

Uncontrolled development and use of the floodplains and rivers of this municipality would impair the public health, safety, convenience, general welfare and tax base.

1.3	STATEMENT OF PURPOSE

This ordinance is intended to regulate floodplain development to:

(1)	Protect life, health and property;

(2)	Minimize expenditures of public funds for flood control projects;

(3)	Minimize rescue and relief efforts undertaken at the expense of the taxpayers;

(4)	Minimize business interruptions and other economic disruptions;

(5)	Minimize damage to public facilities in the floodplain;

(6)	Minimize the occurrence of future flood blight areas in the floodplain;

(7)	Discourage the victimization of unwary land and homebuyers;

(8)	Prevent increases in flood heights that could increase flood damage and result in 	conflicts between property owners; and

(9)	Discourage development in a floodplain if there is any practicable alternative to 	locate the activity, use or structure outside of the floodplain.

1.4	TITLE

This ordinance shall be known as the Floodplain Zoning Ordinance for the Village of Silver Lake, Wisconsin.

1.5	GENERAL PROVISIONS

(1)	AREAS TO BE REGULATED

This ordinance regulates all areas that would be covered by the regional flood or

base flood.  Note:  Base flood elevations are derived from the flood profiles in the 	Flood Insurance Study.  Regional flood elevations may be derived from other 	studies.  Areas covered by the base flood are identified as A-Zones on the Flood 	Insurance Rate Map.

(2)	OFFICIAL MAPS & REVISIONS

The boundaries of all floodplain districts are designated as floodplains or A-Zones on the maps listed below and the revisions in the Village of Silver Lake Floodplain Appendix.  Any change to the base flood elevations (BFE) in the Flood Insurance Study (FIS) or on the Flood Insurance Rate Map (FIRM) must be reviewed and approved by the DNR and FEMA before it is effective.  No changes to regional flood elevations (RFE’s) on non-FEMA maps shall be effective until approved by the DNR.  These maps and revisions are on file in the office of the Village Hall, Village of Silver Lake.  If more than one map or revision is referenced, the most restrictive information shall apply.

OFFICIAL MAPS:  Based on the FIS

(a)	Flood Insurance Rate Map (FIRM), panel number 550210 0005 B, dated 	September 1, 1978; with corresponding profiles that are based on the 	Flood Insurance Study (FIS) dated March 1, 1978.

(b)	Flood Boundary and Floodway Map (FBFW), panel number 550210 	0005B, dated 9/1/78; with corresponding profiles.

OFFICIAL MAPS:  Based on other studies

(a)	Supplementary Floodplain and Shoreland-wetland Zoning Map, prepared 	for Kenosha County, Wisconsin by Aero-Metric Engineering, Inc. under a 	program administered by the Southeastern Wisconsin Regional Planning 	Commission.  Date of photography April 2002, which designates 	Shoreland-wetlands, Floodway, and Floodplain Conservancy and 	Floodfringe districts.

(3)	ESTABLISHMENT OF DISTRICTS

The regional floodplain areas are divided into four districts as follows:

(a)	The Floodway District (FW) is the channel of a river or stream and those 	portions of the	floodplain adjoining the channel required to carry the 	regional floodwaters.

(b)	The Floodplain Conservancy District (FPC) is those areas, but is not 	limited to, all shoreland-wetlands of 5 acres or greater in area.

(c)	The Floodfringe District (FF) is that portion of the floodplain between the 	regional flood limits and floodway.

(d)	The General Floodplain District (GFP) is those areas that have been or 	may be covered by floodwater during the regional flood.

(e)	The Flood Storage District (FSD) is that area of the floodplain where 	storage of floodwaters is calculated to reduce the regional flood discharge.

(4)	LOCATING FLOODPLAIN BOUNDARIES

Discrepancies between boundaries on the official floodplain zoning map and actual field conditions shall be resolved using the criteria in paragraphs (a) or (b) below.  If a significant difference exists, the map shall be amended according to   s.9.0.  The zoning administrator can rely on a boundary derived from a profile elevation to grant or deny a land use permit, whether or not a map amendment is required.  The zoning administrator shall be responsible for documenting actual pre-development field conditions and the basis upon which the district boundary was determined and for initiating any map amendments required under this section.  Disputes between the zoning administrator and an applicant over the district boundary line shall be settled according to s. 8.3(3) and the criteria in (a) and (b) below.

(a)	If flood profiles exist, the map scale and the profile elevations shall 	determine the district boundary.  The regional or base flood elevations 	shall govern if there are any discrepancies.

(b)	Where flood profiles do not exist, the locations of the boundary shall be 	determined by the map scale, visual on-site inspection and any 	information provided by the Department.

Note:  Where the flood profiles are based on established base flood elevations

from a FIRM, FEMA must also approve any map amendment pursuant to s.9.1(6).

(5)	REMOVAL OF LANDS FROM FLOODPLAIN

Compliance with the provisions of this ordinance shall not be grounds for removing land from the floodplain unless it is filled at least two feet above the regional or base flood elevation, the fill is contiguous to land outside the floodplain, and the map is amended pursuant to s. 9.0.

Note:	This procedure does not remove the requirements for the mandatory 	purchase of flood insurance.  The property owner must contact FEMA to 	request a Letter of Map Change (LOMC).

(6)	COMPLIANCE

Any development or use within the areas regulated by this ordinance shall be in compliance with the terms of this ordinance, and other applicable local, state, and federal regulations.

(7)	MUNICIPALITIES AND STATE AGENCIES REGULATED

Unless specifically exempted by law, all cities, villages, towns, and counties are required to comply with this ordinance and obtain all necessary permits.  State agencies are required to comply if s. 13.48(13), Stats., applies.  The construction, reconstruction, maintenance and repair of state highways and bridges by the Wisconsin Department of Transportation is exempt when s. 30.2022, Stats., applies.

(8)	ABROGATION AND GREATER RESTRICTIONS

(a)	This ordinance supersedes all the provisions of any municipal zoning 	ordinance enacted under ss. 59.69, 59.692 or 59.694 for counties; s. 62.23 	for cities; or s. 61.35 for villages; or s. 87.30, Stats., which relate to 	floodplains.  If another ordinance is more restrictive than this ordinance, 	that ordinance shall continue in full force and effect to the extent of the 	greater restrictions, but not otherwise.

(b)	This ordinance is not intended to repeal, abrogate or impair any existing 	deed restrictions, covenants or easements.  If this ordinance imposes 	greater restrictions, the provisions of this ordinance shall prevail.

(9)	INTERPRETATION

In their interpretation and application, the provisions of this ordinance are the minimum requirements liberally construed in favor of the governing body and are not a limitation on or repeal of any other powers granted by the Wisconsin Statutes.  If a provision of the ordinance, required by ch. NR 116, Wis. Adm. Code, is unclear, the provision shall be interpreted in light of the standards in effect on the date of the adoption of this ordinance or in effect on the date of the most recent text amendment to this ordinance.

(10)	WARNING AND DISCLAIMER OF LIABILITY

The flood protection standards in this ordinance are based on engineering experience and scientific research.  Larger floods may occur or the flood height may be increased by man-made or natural causes.  This ordinance does not imply or guarantee that non-floodplain areas or permitted floodplain uses will be free from flooding and flood damages.  Nor does this ordinance create liability on the part of, or a cause of action against, the municipality or any officer or employee thereof for any flood damage that may result from reliance on this ordinance.

(11)	SEVERABILITY

Should any portion of this ordinance be declared unconstitutional or invalid by a court of competent jurisdiction, the remainder of this ordinance shall not be affected.

(12)	ANNEXED AREAS FOR CITIES AND VILLAGES

The Kenosha County floodplain zoning provisions in effect on the date of annexation shall remain in effect and shall be enforced by the municipality for all annexed areas until the municipality adopts and enforces an ordinance which meets the requirements of ch. NR 116, Wis. Adm. Code and the National Flood Insurance Program (NFIP).  These annexed lands are described on the municipality’s official zoning map.  County floodplain zoning provisions are incorporated by reference for the purpose of administering this section and are on file in the office of the municipal zoning administrator.  All plats or maps of annexation shall show the regional flood elevation and the location of the floodway.

(13)	GENERAL DEVELOPMENT STANDARDS

The community shall review all permit applications to determine whether proposed building sites will be reasonably safe from flooding.  If  proposed building site is in a flood-prone area, all new construction and substantial improvements shall be designed or modified and adequately anchored to prevent flotation, collapse, or lateral movement of the structure resulting from hydrodynamic and hydrostatic loads; be constructed with materials resistant to flood damage; be constructed by methods and practices that minimize flood damages; and be constructed with electrical, heating, ventilation, plumbing, and air conditioning equipment and other service facilities designed and/or located so as to prevent water from entering or accumulating within the components during conditions of flooding.  Subdivisions shall be reviewed for compliance with the above standards.  All subdivision proposals (including manufactured home parks) shall include regional flood elevation and floodway data for any development that meets the subdivision definition of this ordinance.

2.0 	GENERAL STANDARDS APPLICABLE TO ALL FLOODPLAINS

2.1	HYDRAULIC AND HYDROLOGIC ANALYSES

(1)	Except as allowed in par. (3) below, no floodplain development shall:

(a)	Obstruct flow, defined as development which blocks the conveyance of 			floodwaters by itself or with other development, increasing regional flood 			height; or

(b)	Increase regional flood height due to floodplain storage area lost, which 			equals or exceeds 0.01 foot.

(2)	The zoning administrator shall deny permits if it is determined the proposed 	development will obstruct flow or increase regional flood heights 0.01 foot or 	more, based on the officially adopted FIRM or other adopted map, unless the 	provisions of sub. (3) are met.

(3)	Obstructions or increases equal to or greater than 0.01 foot may only be permitted 	if amendments are made to this ordinance, the official floodplain zoning maps, 	floodway lines and water surface profiles, in accordance with s. 9.0.

Note:  This section refers to obstructions or increases in base flood elevations as shown on the officially adopted FIRM or other adopted map.  Any such alterations must be reviewed and approved by FEMA and the DNR.

2.2 	WATERCOURSE ALTERATIONS

No land use permit to alter or relocate a watercourse in a mapped floodplain shall be issued until the local official has notified in writing all adjacent municipalities, the Department and FEMA regional offices and required the applicant to secure all necessary state and federal permits.  The flood carrying capacity of any altered or relocated watercourse shall be maintained.  As soon as is practicable, but not later than six months after the date of the watercourse alteration or relocation, the zoning administrator shall notify FEMA of the changes by submitting appropriate technical or scientific data in accordance with NFIP guidelines that shall be used to revise the FIRM, risk premium rates and floodplain management regulations as required.

2.3	CHAPTER 30, 31, WIS. STATS., DEVELOPMENT

Development which requires a permit from the Department, under chs. 30 and 31, Wis. Stats., such as docks, piers, wharves, bridges, culverts, dams and navigational aids, may be allowed if the necessary permits are obtained and amendments to the floodway lines, water surface profiles, BFE’s established in the FIS, or other data from the officially adopted FIRM, or other floodplain zoning maps or the floodplain zoning ordinance are made according to s. 9.0.

2.4	PUBLIC OR PRIVATE CAMPGROUNDS

Public or private campgrounds shall have a low flood damage potential and shall meet the following provisions:

(1)	The campground is approved by the Department of Health and Family Services.

(2)	A land use permit for the campground is issued by the zoning administrator.

(3)	The character of the river system and the elevation of the campground is such that 	a 72 hour warning of an impending flood can be given to all campground 	occupants.

(4)	There is an adequate flood warning procedure for the campground that offers the 	minimum notice required under this section to all persons in the campground.  	This procedure shall include a written agreement between the campground owner, 	the municipal emergency government coordinator and the chief law enforcement 	official which specifies the flood elevation at which evacuation shall occur, 	personnel responsible for monitoring flood elevations, types of warning systems 	to be used and the procedures for notifying at-risk parties, and the methods and 	personnel responsible for conducting the evacuation.

(5)	This agreement shall be for no more than one calendar year, at which time the 	agreement shall be reviewed and updated – by the officials identified in sub. (4) – 	to remain in compliance with all applicable regulations, including those of the 	state department of health and family services and all other applicable regulations.

(6)	Only camping units are allowed.

(7)	The camping units may not occupy any site in the campground for more than 180 	consecutive days, at which time the camping unit must be removed from the 	floodplain for a minimum of 24 hours.

(8)  	All camping units that remain on site for more than 30 days shall be issued a limited authorization by the campground operator, a written copy of which is kept on file at the campground.  Such authorization shall allow placement of a camping unit for a period not to exceed 180 days and shall ensure compliance with all the provisions of this section.

(9)	The municipality shall monitor the limited authorizations issued by the campground operator to assure compliance with the terms of this section.

(10)	All camping units that remain in place for more than 180 consecutive days must meet the applicable requirements in either s. 3.0 or s. 5.0 for the floodplain district in which the structure is located.

(11)	The campground shall have signs clearly posted at all entrances warning of the flood hazard and the procedures for evacuation when a flood warning is issued.

(12)	All service facilities, including but not limited to refuse collection, electrical service, natural gas lines, propane tanks, sewage systems and wells shall be properly anchored and placed at or floodproofed to the flood protection elevation.

3.0	FLOODWAY DISTRICT (FW)

3.1	APPLICABILITY

This section applies to all floodway areas on the floodplain zoning maps and those identified pursuant to s. 6.1(4).

3.2	PERMITTED USES

The following open space uses are allowed in the floodway district and the floodway areas of the general floodplain district, if

--  they are not prohibited by any other ordinance;

--  they meet the standards in s. 3.3 and 3.4; and

--  all permits or certificates have been issued according to s. 8.1:

(1)	Agricultural uses, such as: farming, outdoor plant nurseries, horticulture, 	viticulture and wild crop harvesting.

(2)	Nonstructural industrial and commercial uses, such as loading areas, parking areas 	and airport landing strips.

(3)	Nonstructural recreational uses, such as golf courses, tennis courts, archery 	ranges, picnic grounds, boat ramps, swimming areas, parks, wildlife and nature 	preserves, game farms, fish hatcheries, shooting, trap and skeet activities, hunting 	and fishing areas and hiking and horseback riding trails, subject to the fill 	limitations of s. 3.3(4).

(4)	Uses or structures accessory to open space uses, or classified as historic structures 	that comply with ss. 3.3 and 3.4.

(5)	Extraction of sand, gravel or other materials that comply with s. 3.3(4).

(6)	Functionally water-dependent uses, such as docks, piers or wharves, dams, 	flowage areas, culverts, navigational aids and river crossing of transmission lines, 	and pipelines that comply with chs. 30, 31, Stats.

(7)	Public utilities, streets and bridges that comply with s. 3.3(3).

3.3	STANDARDS FOR DEVELOPMENTS IN FLOODWAY AREAS

(1)	GENERAL

(a)	Any development in floodway areas shall comply with s. 2.0 and have a 		low flood damage potential.

(b)	Applicants shall provide the following data to determine the effects of the 	proposal according to s. 2.1:

1.	A cross-section elevation view of the proposal, perpendicular to 			the watercourse, showing if the proposed development will 				obstruct flow; or

2.	An analysis calculating the effects of this proposal on regional 			flood height.

(c)	The zoning administrator shall deny the permit application if the project 	will increase flood elevations upstream or downstream 0.01 foot or more, 	based on the data submitted for par. (b) above.

(2)	STRUCTURES

Structures accessory to permanent open space uses or functionally dependent on a 	waterfront location may be allowed by permit if the structures comply with the 	following criteria:

(a)	The structure is not designed for human habitation and does not have a 			high flood damage potential;

(b)	it must be anchored to resist flotation, collapse and lateral movement;

(c)	mechanical and utility equipment must be elevated or floodproofed to or 	above the flood protection elevation; and

(d)	it must not obstruct the flow of flood waters or cause any increase in flood 	levels during the occurrence of the regional flood.

(3)	PUBLIC UTILITIES, STREETS AND BRIDGES

Public utilities, streets and bridges may be allowed by permit, if:

(a)	Adequate flood proofing measures are provided to the flood protection 			elevation; and

(b)	Construction meets the development standards of s. 2.1.

(4)	FILLS OR DEPOSITION OF MATERIALS

Fills or deposition of materials may be allowed by permit, if:

(a)	The requirements of s. 2.1 are met;

(b)	No material is deposited in the navigable channel unless a permit is issued 	by the Department pursuant to ch. 30, Stats., and a permit pursuant to  	s. 404 of the Federal Water Pollution Control Act, Amendments of 1972, 	33 U.S.C. 1344 has been issued, if applicable, and the other requirements 	of this section are met;

(c)	The fill or other materials will be protected against erosion by riprap, 	vegetative cover, sheet piling or bulkheading; and

(d)	The fill is not classified as a solid or hazardous material.

3.4	PROHIBITED USES

All uses not listed as permitted uses in s. 3.2 are prohibited, including the following uses:

(1)	Habitable structures, structures with high flood damage potential, or those not 	associated with permanent open-space uses;

(2)	Storing materials that are buoyant, flammable, explosive, injurious to property, 	water quality, or human, animal, plant, fish or other aquatic life:

(3)	Uses not in harmony with or detrimental to uses permitted in the adjoining 	districts;

(4)	Any private or public sewage systems, except portable latrines that are removed 	prior to flooding and systems associated with recreational areas and Department 	approved campgrounds that meet the applicable provisions of local ordinances 	and ch. COMM 83, Wis. Adm. Code.

(5)	Any public or private wells which are used to obtain potable water, except those 	for recreational areas that meet the requirements of local ordinances and chs. NR 	811 and NR 812, Wis. Adm. Code:

(6)	Any solid or hazardous waste disposal sites;

(7)	Any wastewater treatment ponds or facilities, except those permitted under 	s. NR 110.15(3)(b), Wis. Adm. Code;

(8)	Any sanitary sewer or water supply lines, except those to service existing or 	proposed development located outside the floodway which complies with the 	regulations for the floodplain area occupied.

4.0	FLOODPLAIN – CONSERVANCY OVERLAY DISTRICT

This district is intended to preserve, protect and enhance the streams, shorelands, and wetland areas of the Village of Silver Lake.  The preservation, protection and enhancement of these areas will serve to maintain safe and healthful conditions; maintain and improve water quality, both ground and surface; control storm water runoff; protect streambanks from erosion; protect wildlife habitat; protect native plant communities; avoid the location of structures on soils which are generally not suitable for such use; and protect the waterbased recreation resources of the Village.  If any areas located within the Floodplain-Conservancy Overlay District are also located within the Floodway, Floodfringe, General Floodplain, or Flood Storage Districts, applicable standards for those districts shall also apply.

4.1	PERMITTED PRINCIPAL USES

(1)	Hiking, fishing, trapping, swimming, and boating, unless prohibited by other laws 	or ordinances.

(2)	Harvesting of wild crops, such as marsh hay, ferns, moss, wild rice, berries, tree 	fruits, and tree seeds, in a manner that is not injurious to natural reproduction of 	such crops, and that does not involve filling, flooding, draining, dredging, 	ditching, tiling, or excavation.

(3)	Silviculture, including the planting, thinning, and harvesting of timber, provided 	that no filling, flooding, draining, dredging, ditching, tiling, or excavating is done 	except for temporary water level stabilization measures to alleviate abnormally 	wet or dry conditions which would have an adverse impact on silvicultural 	activities if not corrected.

- (4)	Existing agricultural cultivation and pasturing provided such activities do not 	involve extension cultivated areas, extension of or creation of new drainage 	systems, and further provided such activities do not substantially disturb or impair 	the natural fauna, flora, topography, or water regimen and can be accomplished 	without filling, flooding, or artificial drainage of the wetland.

- (5)	Ditching, tiling, dredging, excavating, or filling done to maintain or repair an 	existing drainage system only to the extent necessary to maintain the level of 	drainage required to continue the existing use.

- (6)	The construction and maintenance of piers, docks, observation decks, walkways 	and bicycle and recreation trails provided that no filling, flooding, dredging, 	draining, ditching, tilling, or excavating is done, except limited filling and 	excavating necessary for the installation of pilings.

- (7)	The maintenance, repair, replacement, and reconstruction of existing streets and 	bridges.

- 4.2	CONDITIONAL USES

- (1)	The construction of streets and bridges if necessary for the continuity of the 	village street system, necessary for the provision of essential utility and public 	safety services, or necessary to provide access to permitted open space uses, 	provided that:

- (a)	The street cannot, as a practical matter, be located outside the lowland 			conservancy district;

- (b)	The street is designed and constructed to minimize the adverse impact 			upon the natural functions of the wetland as listed in subsection 9.

- (c)	The street construction activities are to be carried out in the immediate 			area of the roadbed only; and

- (d)	The street shall be designed and constructed for the minimum cross 			section practical to serve the intended use:

(e)	Any filling, flooding, draining, dredging, ditching, tiling or excavating that 		is to be done must be necessary for the construction or maintenance of the 			street.

(2)	The construction and maintenance of non residential buildings used solely in 	conjunction with raising of waterfowl, minnows or other wetland or aquatic 	animals or used solely for some other purpose which is compatible with wetland 	preservation, provided that:

(a)	The building cannot as a practical matter be located outside the Lowland 			Conservancy District;

(b)	The building is not designed for human habitation and does not exceed 			500 square feet in area; and

(c)	Only limited filling or excavating necessary to provide structural support 			is conducted.

(3)	The establishment and development of public and private parks and recreation 	areas, recreation trails, public boat access sites, natural and outdoor education 	areas, historic and scientific areas, wildlife refuges, game preserves, and private 	wildlife habitat areas, provided that:

(a)	Any private recreation or wildlife habitat areas must be used exclusively 			for that purpose:

(b)	Only limited filling and excavating necessary for the development of 			public boat launching ramps, swimming beaches, park shelters or similar 			structures is allowed:

(c)	Construction and maintenance of streets necessary for permitted uses shall 		be in accordance with paragraph one (1) of this subsection; and

(d)	Ditching, excavating, dredging, and dike and dam construction may be 			done in wildlife refuges, game preserves, and private wildlife habitat 			areas, but only for the purpose of improving wildlife habitat or otherwise 			enhance the value of wetlands or other natural resources.

(4)	The construction and maintenance of electric, gas, telephone, water and sewer 	transmission and distribution lines and related facilities by public utilities and 	cooperative associations organized for the purpose of producing or furnishing 	heat, light, power, or water to members provided that:

(a)	The transmission and distribution lines and related facilities cannot as a 			practical matter be located outside the Floodplain Conservancy/Overlay 			District; and

(b)	Any filling, excavating, ditching, or draining that is to be done must be 			necessary for the construction or maintenance of the utility, and must be 			done in a manner designed to minimize flooding and other adverse 				impacts upon the natural functions of the conservancy area.

(5)	The construction and maintenance of railroad lines, provided that:

(a)	The railroad lines cannot as a practical matter be located outside the 			lowland conservancy district; and

(b)	Any filling, excavating, ditching, or draining that is to be done must be 			necessary for the construction or maintenance of the railroad lines, and 			must be done in a manner designed to minimize flooding and other 			adverse impacts upon the natural functions of the Floodplain-Conservancy 		Overly District.

4.3	PROHIBITED USES

(1)	Any use is not listed as a permitted principal use or a conditional use is prohibited 	unless the Floodplain-Conservancy Overly District lands concerned are first 	rezoned into another district.

(2)	The use of a boathouse for human habitation and the construction or placement of 	a boathouse or fixed houseboat below the ordinary high watermark of any 	navigable water is prohibited.

The Village encourages the private preservation of sensitive environmental areas and desires that environmental open space and buffers be utilized whenever possible in all developments and projects; however, the assemblage or acquisition of Floodplain-Conservancy/Overlay District Lands for the express purpose of boosting or increasing the density of development is prohibited.

5.0	FLOODFRINGE DISTRICT (FF)

5.1	APPLICABILITY

This section applies to all floodfringe areas shown on the floodplain zoning maps and those identified pursuant to s. 6.1(4)

5.2	PERMITTED USES

Any structure, land use, or development is allowed in the floodfringe district if the standards in s. 5.3 are met, the use is not prohibited by this or any other ordinance or regulation and all permits or certificates specified in s. 8.1 have been issued.

5.3	STANDARDS FOR DEVELOPMENT IN FLOODFRINGE AREAS

S. 2.1 shall apply in addition to the following requirements according to the use requested.

(1)	RESIDENTIAL USES

Any habitable structure, including a manufactured home, which is to be erected, 	constructed, reconstructed, altered, or moved into the floodfringe area, shall meet 	or exceed the following standards;

(a)	The elevation of the lowest floor, excluding the basement or crawlway, 			shall be at or above the flood protection elevation on fill.  The fill shall be 			one foot or more above the regional flood elevation extending at least 15 			feet beyond the limits of the structure.  The Department may authorize 			other floodproofing measures if the elevations of existing streets or sewer 			lines makes compliance with the fill standards impractical;

(b)	The basement or crawlway floor may be placed at the regional flood 			elevation if it is floodproofed to the flood protection elevation.  No 			basement or crawlway floor is allowed below the regional flood elevation;

(c)	Contiguous dryland access shall be provided from a structure to land 			outside of the floodplain, except as provided in par. (d).

(d)	In developments where existing street or sewer line elevations make 			compliance with par. (c) impractical, the municipality may permit new 			development and substantial improvements where access roads are at or 			below the regional flood elevation, if:

1.	The municipality has written assurance from police, fire and 				emergency services that rescue and relief will be provided to the

structure(s) by wheeled vehicles during a regional flood event; or

2.	The municipality has a natural disaster plan approved by 					Wisconsin Emergency Management and the Department.

(2)	ACCESSORY STRUCTURES OR USES

(a)	Except as provided in par. (b), an accessory structure which is not 				connected to a principal structure may be constructed with its lowest floor 			at or above the regional flood elevation.

(b)	An accessory structure which is not connected to the principal structure 			and which is less than 600 square feet in size and valued at less than 			$10,000 may be constructed with its lowest floor no more than two feet 			below the regional flood elevation if it is subject to flood velocities of no 			more than two feet per second and it meets all of the provisions of sections 		3.3(2)(a),(b),(c) and (d) and 5.3(5) below.

(3)	COMMERCIAL USES

Any commercial structure which is erected, altered or moved into the floodfringe 	area shall meet the requirements of s. 5.3(1).  Subject to the requirements of sub. 	5.3(5), storage yards, surface parking lots and other such uses may be placed at 	lower elevations if an adequate warning system exists to protect life and property.

(4)	MANUFACTURING AND INDUSTRIAL USES

Any manufacturing or industrial structure which is erected, altered or moved into 	the floodfringe area shall be protected to the flood protection elevation using fill, 	levees, floodwalls, or other flood proofing measures in s. 8.5.  Subject to the 	requirements of sub. 5.3(5), storage yards, surface parking lots and other such 	uses may be placed at lower elevations if an adequate warning system exists to 	protect life and property.

(5)	STORAGE OF MATERIALS

Materials that are buoyant, flammable, explosive, or injurious to property, water 	quality of human, animal, plant, fish or aquatic life shall be stored at or above the 	flood protection elevation or floodproofed in compliance with s. 8.5.  Adequate 	measures shall be taken to ensure that such materials will not enter the water body 	during flooding.

(6)	PUBLIC UTILITIES, STREETS AND BRIDGES

All utilities, streets and bridges shall be designed to be compatible with 	comprehensive floodplain development plans; and

(a)	When failure of public utilities, streets and bridges would endanger public 			health or safety, or where such facilities are deemed essential, construction 		of and substantial improvements to such facilities may only be permitted if 		they are floodproofed in compliance with s. 8.5 to the flood protection 			elevation;

(b)	Minor roads or nonessential utilities may be constructed at lower 				elevations if they are designed to withstand flood forces to the regional 			flood elevation.

(7)	SEWAGE SYSTEMS

All on-site sewage disposal systems shall be floodproofed, pursuant to s. 8.5, to 	the flood protection elevation and shall meet the provisions of all local ordinances 	and ch. COMM 83, Wis. Adm. Code.

(8)	WELLS

All wells shall be floodproofed, pursuant to s. 8.5, to the flood protection 	elevation and shall meet the provisions of chs. NR 811 and NR 812, Wis. Adm. 	Code.

(9)	SOLID WASTE DISPOSAL SITES

Disposal of solid or hazardous waste is prohibited in floodfringe areas.

(10)	DEPOSITION OF MATERIALS

Any deposited material must meet all the provisions of this ordinance.

(11)	MANUFACTURED HOMES

(a)	Owners or operators of all manufactured home parks and subdivisions 			shall provide adequate surface drainage to minimize flood damage, and 			prepare, secure approval and file an evacuation plan, indicating vehicular 			access and escape routes, with local emergency management authorities.

(b)	In existing manufactured home parks, all new homes, replacement homes 			on existing pads, and substantially improved homes shall:

1.  have the lowest floor elevated to the flood protection elevation; and

2.  be anchored so they do not float, collapse or move laterally during a 			        flood.

(c)	Outside of existing manufactured home parks, including new 				manufactured home parks and all single units outside of existing parks, all 			new, replacement and substantially improved manufactured homes shall 			meet the residential development standards for the floodfringe in s. 5.3(1).

(12)	MOBILE RECREATIONAL VEHICLES

All mobile recreational vehicles that are on site for 180 consecutive days or more 	or are not fully licensed and ready for highway use shall meet the elevation and 	anchoring requirements in s. 5.3 (11)(b) and (c).  A mobile recreational vehicle is 	ready for highway use if it is on its wheels or jacking system, is attached to the 	site only by quick-disconnect utilities and security devices and has no 	permanently attached additions.

6.0	OTHER FLOODPLAIN DISTRICTS

Other floodplain districts may be established under the ordinance and reflected on the floodplain zoning map.  These districts may include general floodplain districts and flood storage districts.

6.1	GENERAL FLOODPLAIN DISTRICT (GFP)

(1)	APPLICABILITY

The provisions for this district shall apply to all floodplains for which flood 	profiles are not available or where flood profiles are available but floodways have 	not been delineated.  Floodway and floodfringe districts shall be delineated when 	adequate data is available.

(2)	PERMITTED USES

Pursuant to s. 6.1(4) it shall be determined whether the proposed use is located 	within a floodway or floodfringe area.

Those uses permitted in floodway (s. 3.2) and floodfringe areas (s. 5.2) are 	allowed within the general floodplain district, according to the standards of s. 6.3, 	provided that all permits or certificates required under s. 8.1 have been issued.

(3)	STANDARDS FOR DEVELOPMENT IN THE GENERAL FLOODPLAIN   	DISTRICT

S. 3.0 applies to floodway areas, s. 5.0 applies to floodfringe areas.  The rest of 	this ordinance applies to either district.

(4)	DETERMINING FLOODWAY AND FLOODFRINGE LIMITS

Upon receiving an application for development within the general floodplain 	district, the zoning administrator shall.

(a)	Require the applicant to submit two copies of an aerial photograph or a 	plan which shows the proposed development with respect to the general 	floodplain district limits, stream channel, and existing floodplain 	developments, along with a legal description of the property, fill limits and 	elevations, building floor elevations and flood proofing measures:

(b)	Require the applicant to furnish any of the following information deemed 	necessary by the Department to evaluate the effects of the proposal upon 	flood height and flood flows, regional flood elevation and to determine 	floodway boundaries:

(1)	A typical valley cross-section showing the stream channel, the 			floodplain adjoining each side of the channel, the cross-sectional 			area to 	be occupied by the proposed development, and all historic 			high water information;

(2)	Plan (surface view) showing elevations or contours of the ground; 			pertinent structure, fill or storage elevations; size, location and 			layout of all proposed and existing structure on the site; location 			and elevations of streets, water supply, and sanitary facilities; soil			types and other pertinent information;

(3)	Profile showing the slope of the bottom of the channel or flow line 			of the stream;

(4)	Specifications for building construction and materials, 				floodproofing, filling, dredging, channel improvement, storage, 			water supply and sanitary facilities.

(c)	Transmit one copy of the information described in pars. (1) and (2) to the 	Department Regional office along with a written request for technical 	assistance to establish regional flood elevations and, where applicable, 	floodway data.  Where the provisions of s. 8.1 (2)(c) apply, the applicant 	shall provide all required information and computations to delineate 	floodway boundaries and the effects of the project on flood elevations.

6.2	FLOOD STORAGE DISTRICT

The flood storage district delineates that portion of the floodplain where storage of floodwaters has been taken into account and is relied upon to reduce the regional flood discharge.  The district protects the flood storage areas and assures that any development in the storage areas will not decrease the effective flood storage capacity which would cause higher flood elevations.

(1)	APPLICABILITY

The provisions of this section apply to all areas within the Flood Storage District 	(FSD), as shown on the official floodplain zoning maps.

(2)	PERMITTED USES

Any use or development which occurs in a flood storage district must meet the 	applicable requirements in s. 5.3.

(3)	STANDARDS FOR DEVELOPMENT IN FLOOD STORAGE DISTRICTS

(a)	Development in a flood storage district shall not cause an increase equal or 	greater than 0.01 of a foot in the height of the regional flood.

(b)	No development shall be allowed which removes flood storage volume 	unless an equal volume of storage as defined by the pre-development 	ground surface and the regional flood elevation shall be provided in the 	immediate area of the proposed  development to compensate for the 	volume of storage which is lost, (compensatory storage).  Excavation 	below the groundwater table is not considered to provide an equal volume 	of storage.

(c)	If compensatory storage cannot be provided, the area may not be 	developed unless the entire area zoned as flood storage district – on this 	waterway – is rezoned to the floodfringe district.  This must include a 	revision to the floodplain study and map done for the waterway to revert 	to the higher regional flood discharge calculated without floodplain 	storage, as per s. 9.1 of this ordinance.

(d)	No area may be removed from the flood storage district unless it can be 	shown that the area has been filled to the flood protection elevation and is 	contiguous to other lands lying outside of the floodplain.

7.0	NONCONFORMING USES

7.1	GENERAL

(1)	APPLICABILITY

If these standards conform with s. 59.69(10), Stats., for counties or s. 62.23(7)(h), 	Stats., for cities and villages, they shall apply to all modifications or additions to 	any nonconforming use or structure and to the use of any structure or premises 	which was lawful before the passage of this ordinance or any amendment thereto.

(2)	The existing lawful use of the structure or its accessory use which is not in 	conformity with the provisions of this ordinance may continue subject to the 	following conditions:

(a)	No modifications or additions to a nonconforming use or structure shall be 	permitted unless they comply with this ordinance.  The words 	“modification” and “addition” include, but are not limited to, any 	alteration, addition, modification, structural repair, rebuilding or 	replacement of any such existing use, structure or accessory structure or 	use.  Ordinary maintenance repairs are not considered an extension, 	modification or addition; these include painting, decorating, paneling and 	the replacement of doors, windows and other nonstructural components 	and the maintenance, repair or replacement of existing private sewage or 	water supply systems or connections to public utilities.  Ordinary 	maintenance repairs do not include any costs associated with the repair of 	a damaged structure.

The construction of a deck that does not exceed 200 square feet and that is 	adjacent to the exterior wall of a principal structure is not an extension, 	modification or addition.  The roof of the structure may extend over a 	portion of the deck in order to provide safe ingress and egress to the 	principal structure.

(b)	If a nonconforming use or the use of a nonconforming structure is 	discontinued for 12 consecutive months, it is no longer permitted and any 	future use of the property, and any structure or building thereon, shall 	conform to the applicable requirements of this ordinance;

(c)	The municipality shall keep a record which lists all nonconforming uses 	and nonconforming structures, their present equalized assessed value, the 	cost of all modifications or additions which have been permitted, and the 	percentage of the structure’s total current value those modifications 	represent;

(d)	No modification or addition to any nonconforming structure or any 	structure with a nonconforming use, which over the life of the structure 	would exceed 50% of its present equalized assessed value, shall be 	allowed unless the entire structure is permanently changed to a 	conforming structure with a conforming use in compliance with the 	applicable requirements of this ordinance.  Contiguous dry land access 	must be provided for residential and commercial uses in compliance with 	s. 5.3(1).  The costs of elevating a nonconforming building or a building 	with a nonconforming use to the flood protection elevation are excluded 	from the 50% provisions of this paragraph;

(e)	1.  Except as provided in subd. 2., if any nonconforming structure or any 	structure with a nonconforming use is destroyed or is substantially 	damaged, it cannot be replaced, reconstructed or rebuilt unless the use 	and the structure meet the current ordinance requirements.  A structure is 	considered substantially damaged if the total cost to restore the structure to 	its pre-damaged condition exceeds 50% of the structure’s present 	equalized assessed value.

2.  For nonconforming buildings that are damaged or destroyed by a 	nonflood disaster, the repair or reconstruction of any such nonconforming 	building may be permitted in order to restore it after the nonflood disaster, 	provided that the nonconforming building will meet all the minimum 	requirements under 44 CFR Part 60, or under the regulations promulgated 	thereunder.

(f)	A nonconforming historic structure may be altered if the alteration will not 	preclude the structures continued designation as a historic structure, the 	alteration will comply with s. 3.3(1), flood resistant materials are used, 	and the construction practices and floodproofing methods that comply 	with s. 8.5 are used.

7.2	FLOODWAY AREAS

(1)	No modification or addition shall be allowed to any nonconforming structure or 	any structure with a nonconforming use in a floodway area, unless such 	modification or addition:

(a)	Has been granted a permit or variance which meets all ordinance 				requirements;

(b)	Meets the requirements of s. 7.1;

(c)	Will not increase the obstruction to flood flows or regional flood height; 			and

(d)	Any addition to the existing structure shall be floodproofed, pursuant to  			s. 8.5, by means other than the use of fill, to the flood protection elevation.

(e)	If any part of the foundation below the flood protection elevation is 			enclosed, the following standards shall apply:

1.	The enclosed area shall be designed by a registered architect or 			engineer to allow for the efficient entry and exit of flood waters 			without human intervention.  A minimum of two openings must be 		provided with a minimum net area of at least one square inch for 			every one square foot of the enclosed area.  The lowest part of the 			opening can be no more than 12 inches above the adjacent grade;

2.	The parts of the foundation located below the flood protection 			elevation must be constructed of flood-resistant materials.

3.	Mechanical and utility equipment must be elevated or floodproofed 		to or above the flood protection elevation; and

4.	The use must be limited to parking or limited storage.

(2)	No new on-site sewage disposal system, or addition to an existing on-site sewage 	disposal system, except where an addition has been ordered by a government 	agency to correct a hazard to public health, shall be allowed in a floodway area.  	Any replacement, repair or maintenance of an existing on-site sewage disposal 	system in a floodway area shall meet the applicable requirements of all municipal 	ordinances and ch. COMM 83, Wis., Adm. Code.

(3)	No new well or modification to an existing well used to obtain potable water shall 	be allowed in a floodway area.  Any replacement, repair or maintenance of an 	existing well in floodway area shall meet the applicable requirements of all 	municipal ordinances and chs. NR 811 and NR 812, Wis. Adm. Code.

7.3	FLOODFRINGE AREAS

(1)	No modification or addition shall be allowed to any nonconforming structure or 	any structure with a nonconforming use unless such modification or addition has 	been granted a permit or variance by the municipality, and the modification or 	addition shall be placed on fill or floodproofed to the flood protection elevation in 	compliance with the standards for that particular use in s. 5.3 and 8.5, except 	where s. 7.3(2) is applicable.

(2)	Where compliance with provisions of par. (1) would result in unnecessary 	hardship and only where the structure will not be used for human habitation or be 	associated with a high flood damage potential, the Board of Adjustment/Appeals, 	using the procedures established in s. 8.3, may grant a variance from those 	provisions of par. (1) for modifications or additions, using the criteria listed 	below.  Modifications or additions which are protected to elevations lower than 	the flood protection elevation may be permitted if:

(a)	No floor is allowed below the regional flood elevation for residential or 			commercial structures;

(b)	Human lives are not endangered;

(c)	Public facilities, such as water or sewer, will not be installed;

(d)	Flood depths will not exceed two feet;

(e)	Flood velocities will not exceed two feet per second; and

(f)	The structure will not be used for storage of materials as described in

s. 5.3(6).

(3)	If neither the provisions of par. (1) or (2) above can be met, one addition to an 	existing room in a nonconforming building or a building with a nonconforming 	use may be allowed in the floodfringe, if the addition:

(a)	Meets all other regulations and will be granted by permit or variance;

(b)	Does not exceed 60 square feet in area; and

(c)	In combination with other previous modifications or additions to the 			building, does not exceed 50% of the present equalized assessed value of 			the building.

(4)	All new private sewage disposal systems, or addition to, replacement, repair or 	maintenance of a private sewage disposal system shall meet all the applicable 	provisions of all local ordinances and ch. COMM 83, Wis. Adm. Code.

(5)	All new wells, or addition to, replacement, repair or maintenance of a well shall 	meet the applicable provisions of this ordinance and ch. NR811 and NR812, 	Wis. Adm. Code.

7.4	FLOOD STORAGE AREAS

No modifications or additions shall be allowed to any nonconforming structure in a flood storage area unless the standards outlined in 6.2(3) are met.

8.0	ADMINISTRATION

Where a zoning administrator, planning agency or a board of adjustment/appeals has already been appointed to administer a zoning ordinance adopted under ss. 59.69, 59.692 or 62.23(7), Stats, these officials shall also administer this ordinance.

8.1	ZONING ADMINISTRATOR

(1)	The zoning administrator is authorized to administer this ordinance and shall have 	the following duties and powers:

(a)	Advise applicants of the ordinance provisions, assist in preparing permit 			applications and appeals, and assure that the regional flood elevation for 			the proposed development is shown on all permit applications.

(b)	Issue permits and inspect properties for compliance with provisions of this 		ordinance, and issue certificates of compliance where appropriate.

(bm)	Inspect all damaged floodplain structures and perform substantial damage 			assessment to determine if substantial damage to the structures has 				occurred.

(c)	Keep records of all official actions such as:

1.	All permits issued, inspections made, and work approved;

2.	Documentation of certified lowest floor and regional flood 					elevations for floodplain development;

3.	Records of water surface profiles, floodplain zoning maps and 				ordinances, nonconforming uses and structures including changes, 				appeals, variances and amendments.

4.	All substantial damage assessment reports for floodplain 					structures.

(d)	Submit copies of the following items to the Department Regional office:

1.	Within 10 days of the decision, a copy of any decisions on 					variances, appeals for map or text interpretations, and map or text 				amendments;

2.	Copies of any case-by-case analyses, and any other information 				required by the Department including an annual summary of the 				number and types of floodplain zoning actions taken.

3.	Copies of substantial damage assessments performed and all 				related correspondence concerning the assessments.

Note:  Information on conducting substantial damage assessments is 			available on the DNR website –

http://dnr.wi.gov/org/water/wm/dsfm/flood/title.htm

(e)	Investigate, prepare reports, and report violations of this ordinance to the 			municipal zoning agency and attorney for prosecution.  Copies of the 			reports shall also be sent to the Department Regional office.

(f)	Submit copies of text and map amendments and biennial reports to the 			FEMA Regional Office.

(2)	LAND USE PERMIT

A land use permit shall be obtained before any new development or any repair or 	change in the use of a building or structure, including sewer and water facilities, 	may be initiated.  Application to the zoning administrator shall include:

(a)	GENERAL INFORMATION

1.	Name and address of the applicant, property owner and contractor;

2.	Legal description, proposed use, and whether it is new construction 			or a modification;

(b)	SITE DEVELOPMENT PLAN

A site plan drawn to scale shall be submitted with the permit application 			form and shall contain:

1.	Location, dimensions, area and elevation of the lot;

2.	Location of the ordinary highwater mark of any abutting navigable 				waterways;

3.	Location of any structures with distances measured from the lot 				lines and street center lines;

4.	Location of any existing or proposed on-site sewage systems or 				private water supply systems;

5.	Location and elevation of existing or future access roads;

6.	Location of floodplain and floodway limits as determined from the 				official floodplain zoning maps;

7.	The elevation of the lowest floor of proposed buildings and any fill 			using National Geodetic and Vertical Datum (NGVD);

8.	Data sufficient to determine the regional flood elevation in NGVD 				at the location of the development and to determine whether or not 				the requirements of s. 3.0 or 5.0 are met; and

9.	Data to determine if the proposed development will cause an 				obstruction to flow or an increase in regional flood height or 				discharge according to s. 2.1.  This may include any of the 					information noted in s. 3.3(1).

(c)	DATE REQUIREMENTS TO ANALYZE DEVELOPMENTS

1.	The applicant shall provide all survey data and computations 				required to show the effects of the project on flood heights, 					velocities and floodplain storage, for all subdivision proposals, as 				“subdivision” is defined in s. 236, Stats., and other proposed 				developments exceeding 5 acres in area or where the estimated 				cost exceeds $125,000.  The applicant shall provide:

a.	An analysis of the effect of the development on the regional 				flood profile, velocity of flow and floodplain storage  					capacity;

b.	A map showing location and details of vehicular access to 					lands outside the floodplain; and

c.	A surface drainage plan showing how flood damage will be 				minimized.

The estimated cost of the proposal shall include all structural 				development, landscaping, access and road development, utilities, 				and other pertinent items, but need not include land costs.

(d)	EXPIRATION

All permits issued under the authority of this ordinance shall expire 180 			days after issuance.

(3)	CERTIFICATE OF COMPLIANCE

No land shall be occupied or used, and no building which is hereafter constructed, 	altered, added to, modified, repaired, rebuilt or replaced shall be occupied until a 	certificate of compliance is issued by the zoning administrator, except where no 	permit is required, subject to the following provisions:

(a)	The certificate of compliance shall show that the building or premises or 			part thereof, and the proposed use, conform to the provisions of this 			ordinance;

(b)	Application for such certificate shall be concurrent with the application for 		a permit;

(c)	If all ordinance provisions are met, the certificate of compliance shall be 			issued within 10 days after written notification that the permitted work is 			completed;

(d)	The applicant shall submit a certification signed by a registered 				professional engineer, architect or land surveyor that the fill, lowest floor 			and floodproofing elevations are in compliance with the permit issued.  			Floodproofing measures also require certification by a registered 				professional engineer or architect that floodproofing measures meet the 			requirements of s. 8.5.

(4)	OTHER PERMITS

The applicant must secure all necessary permits from federal, state, and local 	agencies, including those required by the U.S. Army Corps of Engineers under  	s. 404 of the Federal Water Pollution Control Act, Amendments of 1972, 33 	U.S.C. 	1344.

8.2	ZONING AGENCY

(1)	The Zoning Board of Appeals shall:

(a)	oversee the functions of the office of the zoning administrator; and

(b)	review and advise the Governing body on all proposed amendments to this 		ordinance, maps and text.

(2)	This zoning agency shall not:

(a)	grant variances to the terms of the ordinance in place of action by the 			Board of Adjustment/Appeals; or

(b)	amend the text or zoning maps in place of official action by the Governing 		body.

8.3	BOARD OF ADJUSTMENT/APPEALS

The Board of Adjustment/Appeals, created under s. 59.694, Stats., for counties or s.62.23(7)(e), Stats., for cities or villages, is hereby authorized or shall be appointed to act for the purposes of this ordinance.  The Board shall exercise the powers conferred by Wisconsin Statutes and adopt rules for the conduct of business.  The zoning administrator may not be the secretary of the Board.

(1)	POWERS AND DUTIES

The Board of Adjustment/Appeals shall:

(a)	Appeals – Hear and decide appeals where it is alleged there is an error in 			any order, requirement, decision or determination made by an 				administrative official in the enforcement of administration of this 				ordinance.

(b)	Boundary Disputes – Hear and decide disputes concerning the district 			boundaries shown on the official floodplain zoning map.

(c)	Variances – Hear and decide, upon appeal, variances from the ordinance 			standards.

(2)	APPEALS TO THE BOARD

(a)	Appeals to the board may be taken by any person aggrieved, or by any 			officer or department of the municipality affected by any decision of the 			zoning administrator or other administrative officer.  Such appeal shall be 			taken within 30 days unless otherwise provided by the rules of the board, 			by filing with the official whose decision is in question, and with the 			board, a notice of appeal specifying the reasons for the appeal.  The 			official whose decision is in question shall transmit to the board all records 		regarding the matter appealed.

(b)	NOTICE AND HEARING FOR APPEALS INCLUDING VARIANCES

1.	Notice – The board shall:

a.	Fix a reasonable time for the hearing;

b.	Publish adequate notice pursuant to Wisconsin Statutes, 					specifying the date, time, place and subject of the hearing;

c.	Assure that notice shall be mailed to the parties in interest 					and the Department Regional office at least 10 days in 					advance of the hearing.

2.	Hearing – Any party may appear in person or by agent.  The board 				shall:

a.	Resolve boundary disputes according to s. 8.3(3).

b.	Decide variance applications according to s. 8.3(4).

c.	Decide appeals of permit denials according to s. 8.4.

(c)	DECISION:  The final decision regarding the appeal or variance 				application shall:

1.	Be made within a reasonable time;

2.	Be sent to the Department Regional office within 10 days of the 				decision;

3.	Be a written determination signed by the chairman or secretary of 				the Board;

4.	State the specific facts which are the basis for the Board’s 					decision;

5.	Either affirm, reverse, vary or modify the order, requirement, 				decision or determination appealed, in whole or in part, dismiss the 			appeal for lack of jurisdiction or grant or deny the variance 					application;

6.	Include the reasons for granting an appeal, describing the hardship 				demonstrated by the applicant in the case of a variance, clearly 				stated in the recorded minutes of the Board proceedings.

(3)	BOUNDARY DISPUTES

The following procedure shall be used by the Board in hearing disputes 	concerning floodplain district boundaries:

(a)	If a floodplain district boundary is established by approximate or detailed 			floodplain studies, the flood elevations or profiles shall prevail in locating 			the boundary.  If none exist, other evidence may be examined.

(b)	In all cases, the person contesting the boundary location shall be given a 			reasonable opportunity to present arguments and technical evidence to the 			Board.

(c)	If the boundary is incorrectly mapped, the Board should inform the zoning 		committee or the person contesting the boundary location to petition the 			governing body for a map amendment according to s. 9.0.

(4)	VARIANCE

(a)	The Board may, upon appeal, grant a variance from the standards of this 			ordinance if an applicant convincingly demonstrates that:

1.	Literal enforcement of the ordinance provisions will cause 					unnecessary hardship;

2.	The hardship is due to adoption of the floodplain ordinance and 				unique property conditions, not common to adjacent lots or 					premises.  In such case the ordinance or map must be amended;

3.	The variance is not contrary to the public interest; and

4.	The variance is consistent with the purpose of this ordinance in 				s.1.3.

(b)	In addition to the criteria in par. (a), to qualify for a variance under FEMA 		regulations, the following criteria must be met:

1.	The variance may not cause any increase in the regional flood 				elevation;

2.	Variances can only be granted for lots that are less than one-half 				acre and are contiguous to existing structures constructed below 				the RFE;

3.	Variances shall only be granted upon a showing of good and 				sufficient cause, shall be the minimum relief necessary, shall not 				cause increased risks to public safety or nuisances, shall not 				increase costs for rescue and relief efforts and shall not be contrary 			to the purpose of the ordinance.

(c)	A variance shall not:

1.	Grant, extend or increase any use prohibited in the zoning district.

2.	Be granted for a hardship based solely on an economic gain or 				loss.

3.	Be granted for a hardship which is self-created.

4.	Damage the rights or property values of other persons in the area.

5.	Allow actions without the amendments to this ordinance or map(s) 				required in s. 9.1

6.	Allow any alteration of an historic structure, including its use, 				which would preclude its continued designation as an historic 				structure.

(d)	When a floodplain variance is granted the Board shall notify the applicant 			in writing that it may increase flood insurance premiums and risks to life 			and property.  A copy shall be maintained with the variance record.

8.4	TO REVIEW APPEALS OF PERMIT DENIALS

(1)	The Zoning Agency (s. 8.2) or Board shall review all data related to the appeal.

This may include:

(a)	Permit application data listed in s. 8.1(2).

(b)	Floodway/floodfringe determination data in s. 6.1(4).

(c)	Data listed in s. 3.3(1)(b) where the applicant has not submitted this 			information to the zoning administrator

(d)	Other data submitted with the application, or submitted to the Board with 			the appeal.

(2)	For appeals of all denied permits the Board shall:

(a)	Follow the procedures of s. 8.3;

(b)	Consider zoning agency recommendations; and

(c)	Either uphold the denial or grant the appeal.

(3)	For appeals concerning increases in regional flood elevation the Board shall:

(a)	Uphold the denial where the Board agrees with the data showing an 			increase in flood elevation.  Increases equal to or greater than 0.01 foot 			may only be allowed after amending the flood profile and map and all 			appropriate legal arrangements are made with all adversely affected 			property owners.

(b)	Grant the appeal where the Board agrees that the data properly 				demonstrates that the project does not cause an increase equal to or greater 		than 0.01 foot provided no other reasons for denial exist.

8.5	FLOODPROOFING

(1)	No permit or variance shall be issued until the applicant submits a plan certified 	by a registered professional engineer or architect that the floodproofing measures 	will protect the structure or development to the flood protection elevation.

(2)	Floodproofing measures shall be designed to:

(a)	Withstand flood pressures, depths, velocities, uplift and impact forces and 			other regional flood factors;

(b)	Protect structures to the flood protection elevation;

(c)	Anchor structures to foundations to resist flotation and lateral movement; 			and

(d)	Insure that structural walls and floors are watertight to the flood protection 		elevation, and the interior remains completely dry during flooding without 		human intervention.

(3)	Floodproofing measures could include:

(a)	Reinforcing walls and floors to resist rupture or collapse caused by water 			pressure or floating debris.

(b)	Adding mass or weight to prevent flotation.

(c)	Placing essential utilities above the flood protection elevation.

(d)	Installing surface or subsurface drainage systems to relieve foundation 			wall and basement floor pressures;

(e)	Constructing water supply wells and waste treatment systems to prevent 			the entry of flood waters.

(f)	Putting cutoff valves on sewer lines or eliminating gravity flow basement 			drains.

8.6	PUBLIC INFORMATION

(1)	Place marks on structures to show the depth of inundation during the regional 	flood.

(2)	All maps, engineering data and regulations shall be available and widely 	distributed.

(3)	All real estate transfers should show what floodplain zoning district any real 	property is in.

9.0	AMENDMENTS

9.1	GENERAL

The governing body may change or supplement the floodplain zoning district boundaries and this ordinance in the manner provided by law.  Actions which require an amendment include, but are not limited to, the following:

(1)	Any change to the official floodplain zoning map, including the floodway line or 	boundary of any floodplain area.

(2)	Correction of discrepancies between the water surface profiles and floodplain 	zoning maps.

(3)	Any fill in the floodplain which raises the elevation of the filled area to a height at 	or above the flood protection elevation and is contiguous to land lying outside the 	floodplain.

(4)	Any fill or floodplain encroachment that obstructs flow, increasing regional flood 	height 0.01 foot or more.

(5)	Any upgrade to a floodplain zoning ordinance text required by s. NR 116.05, Wis. 	Adm. Code, or otherwise required by law, or for changes by the municipality.

(6)	All channel relocations and changes to the maps to alter floodway lines or to 	remove an area from the floodway or the floodfringe that is based on a base flood 	elevation from a FIRM requires prior approval by FEMA.

Note:  Consult the FEMA web site – www. fema.gov – for the map change fee 	schedule.

9.2	PROCEDURES

Ordinance amendments may be made upon petition of any interested party according to the provisions of s. 62.23, Stats., for cities and villages, or 59.69, Stats., for counties.  Such petitions shall include necessary data required by ss. 6.1(4) and 8.1(2).

(1)	The proposed amendment shall be referred to the zoning agency for a public 	hearing and recommendation to the governing body.  The amendment and notice 	of public hearing shall be submitted to the Department Regional office for review 	 	prior to the hearing.  The amendment procedure shall comply with the provisions 	of s. 62.23, Stats., for cities and villages or s. 59.69, Stats., for counties.

(2)	No amendments shall become effective until reviewed and approved by the 	Department.

(3)	All persons petitioning for a map amendment that obstructs flow, increasing 	regional flood height 0.01 foot or more, shall obtain flooding easements or other 	appropriate legal arrangements from all adversely affected property owners and 	notify local units of government before the amendment can be approved by the 	governing body.

(4)	For amendments in areas with no water surface profiles, the zoning agency or 	board shall consider data submitted by the Department, the zoning administrator’s 	visual on-site inspections and other available information. (See s. 1.5(4).)

10.0	ENFORCEMENT AND PENALTIES

Any violation of the provisions of this ordinance by any person shall be unlawful and shall be referred to the municipal attorney who shall expeditiously prosecute all such violators.  A violator shall, upon conviction, forfeit to the municipality a penalty of not less than $50.00 and not more than $200.00, together with a taxable cost of such action.  Each day of continued violation shall constitute a separate offense.  Every violation of this ordinance is a public nuisance and the creation may be enjoined and the maintenance may be abated by action at suit of the municipality, the state, or any citizen thereof pursuant to s. 87.30, Stats.

11.0	DEFINITIONS

Unless specifically defined, words and phrases in this ordinance shall have their common law meaning and shall be applied in accordance with their common usage.  Words used in the present tense include the future, the singular number includes the plural and the plural number includes the singular.  The word “may” is permissive; “shall” is mandatory and is not discretionary.

1)	“A ZONES” – Those areas shown on the Official Floodplain Zoning Map which would be inundated by the regional flood.  These areas may be numbered or unnumbered A Zones.  The A Zones may or may not be reflective of flood profiles, depending on the availability of data for a given area.

2)	“ACCESSORY STRUCTURE OR USE” – A facility, structure, building or use which is accessory or incidental to the principal use of a property, structure or building.

3)	“BASE FLOOD” – Means the flood having a one percent chance of being equaled or exceeded in any given year, as published by FEMA as part of a FIS and depicted on a FIRM.

4)	“BASEMENT” – Any enclosed area of a building having its floor sub-grade, i.e., below ground level, on all sides.

5)	“BUILDING” – See STRUCTURE.

6)	“BULKHEAD LINE” – A geographic line along a reach of navigable water that has been adopted by a municipal ordinance and approved by the Department pursuant to s. 30.11, Stats., and which allows limited filling between this bulkhead line and the original ordinary highwater mark, except where such filling is prohibited by the floodway provisions of this ordinance.

7)	“CAMPGROUND” – Any parcel of land which is designed, maintained, intended or used for the purpose of providing sites for nonpermanent overnight use by 4 or more camping units, or which is advertised or represented as a camping area.

8)	“CAMPING UNIT” – Any portable device, no more than 400 square feet in area, used as a temporary shelter, including but not limited to a camping trailer, motor home, bus, van, pick-up truck, tent or other mobile recreational vehicle.

9)	“CERTIFICATE OF COMPLIANCE” – A certification that the construction and the use of land or a building, the elevation of fill or the lowest floor of a structure is in compliance with all of the provisions of this ordinance.

10)	“CHANNEL” – A natural or artificial watercourse with definite bed and banks to confine and conduct normal flow or water.

11)	“CRAWLWAYS” OR “CRAWLSPACE” – An enclosed area below the first usable floor of a building, generally less than five feet in height, used for access to plumbing and electrical utilities.

12)	“DECK” – An unenclosed exterior structure that has no roof or sides, but has a permeable floor which allows the infiltration of precipitation.

13)	“DEPARTMENT” – The Wisconsin Department of Natural Resources.

14)	“DEVELOPMENT” – Any artificial change to improved or unimproved real estate, including, but not limited to, the construction of building, structures or accessory structures; the repair of any damaged structure or the improvement or renovation of any structure, regardless of percentage of damage or improvement; the placement of buildings or structures; subdivision layout and the site preparation; mining, dredging, filling, grading, paving, excavation or drilling operations; the storage, deposition or extraction of materials or equipment; and the installation, repair or removal of public or private sewage disposal systems or water supply facilities.

15)	“DRYLAND ACCESS” – A vehicular access route which is above the regional flood elevation and which connects land located in the floodplain to land outside the floodplain, such as a road with its surface above regional flood elevation and wide enough for wheeled rescue and relief vehicles.

16)	“ENCROACHMENT” – Any fill, structure, equipment, building, use or development in the floodway.

17)	“EXISTING MANUFACTURED HOME PARK OR SUBDIVISION” – A parcel of land, divided into two or more manufactured home lots for rent or sale, on which the construction of facilities for servicing the lots is completed before the effective date of this ordinance.  At a minimum, this would include the installation of utilities, the construction of streets and either final site grading or the pouring of concrete pads.

18)	“EXPANSION TO EXISTING MOBILE/MANUFACTURED HOME PARK” – The preparation of additional sites by the construction of facilities for servicing the lots on which the manufactured homes are to be affixed.  This includes installation of utilities, construction of streets and either final site grading, or the pouring of concrete pads.

19)	“FEDERAL EMERGENCY MANAGEMENT AGENCY (FEMA)” – The federal agency that administers the National Flood Insurance Program.

20)	“FLOOD INSURANCE RATE MAP” (FIRM) – A map of a community on which the Federal Insurance Administration has delineated both special flood hazard areas (the floodplain) and the risk premium zones applicable to the community.  This map can only be amended by the Federal Emergency Management Agency.

21)	“FLOOD” or “FLOODING” – A general and temporary condition of partial or complete inundation of normally dry land areas caused by one of the following conditions:

- The overflow or rise of inland waters,

- The rapid accumulation or runoff of surface waters from any source,

- The inundation caused by waves or currents of water exceeding anticipated cyclical levels along the shore of Lake Michigan or Lake Superior, or

- The sudden increase caused by an unusually high water level in a natural body of water, accompanied by a severe storm, or by an unanticipated force of nature, such as a seiche, or by some similarly unusual event.

22)	“FLOOD FREQUENCY” – The probability of a flood occurrence which is determined from statistical analyses.  The frequency of a particular flood event is usually expressed as occurring, on the average, once in a specified number of years or as a percent (%) chance of occurring in any given year.

23)	“FLOODFRINGE” – That portion of the floodplain outside of the floodway which is covered by flood waters during the regional flood and associated with standing water rather than flowing water.

24)	“FLOOD HAZARD BOUNDARY MAP” – A map designating approximate flood hazard areas.  Flood hazard areas are designated as unnumbered A-Zones and do not contain floodway lines or regional flood elevations.  This map forms the basis for both the regulatory and insurance aspects of the National Flood Insurance Program (NFIP) until superseded by a Flood Insurance Study and a Flood Insurance Rate Map.

25)	“FLOOD INSURANCE STUDY” – A technical engineering examination, evaluation, and determination of the local flood hazard areas.  It provides maps designating those areas affected by the regional flood and provides both flood insurance rate zones and base flood elevations and may provide floodway lines.  The flood hazard areas are designated as numbered and unnumbered A-Zones.  Flood Insurance Rate Maps, that accompany the Flood Insurance Study, form the basis for both the regulatory and the insurance aspects of the National Flood Insurance Program.

26)	“FLOODPLAIN” – Land which has been or may be covered by flood water during the regional flood.  It includes the floodway and the floodfringe, and may include other designated floodplain areas for regulatory purposes.

27)	“FLOODPLAIN ISLAND” – A natural geologic land formation within the floodplain that is surrounded, but not covered, by floodwater during the regional flood.

28)	“FLOODPLAIN MANAGEMENT” – Policy and procedures to insure wise use of floodplains, including mapping and engineering, mitigation, education, and administration and enforcement of floodplain regulations.

29)	“FLOOD PROFILE” – A graph or a longitudinal profile line showing the relationship of the water surface elevation of a flood event to locations of land surface elevations along a stream or river.

30)	“FLOODPROOFING” – Any combination of structural provisions, changes or adjustments to properties and structures, water and sanitary facilities and contents of buildings subject to flooding, for the purpose of reducing or eliminating flood damage.

31)	“FLOOD PROTECTION ELEVATION” – An elevation of two feet of freeboard above the water surface profile elevation designated for the regional flood.  (Also see:  FREEBOARD)

32)	“FLOOD STORAGE” – Those floodplain areas where storage of floodwaters has been taken into account during analysis in reducing the regional flood discharge.

33)	“FLOODWAY” – The channel of a river or stream and those portions of the floodplain adjoining the channel required to carry the regional flood discharge.

34)	“FREEBOARD” – A safety factor expressed in terms of a specified number of feet above a calculated flood level.  Freeboard compensates for any factors that cause flood heights greater than those calculated, including ice jams, debris accumulation, wave action, obstruction of bridge openings and floodways, the effects of watershed urbanization, loss of flood storage areas due to development and aggregation of the river or streambed.

35)	“HABITABLE STRUCTURE” – Any structure or portion thereof used or designed for human habitation.

36)	“HEARING NOTICE” – Publications or posting meeting the requirements of Ch. 985, Stats.  For appeals, a Class 1 notice, published once at least one week (7days) before the hearing, is required.  For all zoning ordinances and amendments, a Class 2 notice, published twice, once each week consecutively, the last at least a week (7 days) before the hearing.  Local ordinances or bylaws may require additional notice, exceeding these minimums.

37)	“HIGH FLOOD DAMAGE POTENTIAL” – Damage that could result from flooding that includes any danger to life or health or any significant economic loss to a structure or building and its contents.

38)	“HISTORIC STRUCTURE” – Any structure that is either:

- Listed individually in the National Register of Historic Places or preliminarily determined by the Secretary of the Interior as meeting the requirements for the individual listing on the National Register,

- Certified or preliminarily determined by the Secretary of the Interior as contributing to the historical significance of a registered historic district preliminarily determined by the Secretary to qualify as a registered historic district,

- Individually listed on a state inventory of historic places in states with historic preservation programs which have been approved by the Secretary of the Interior, or

- Individually listed on a local inventory of historic places in communities with historic preservation programs that have been certified either by an approved state program, as determined by the Secretary of the Interior; or by the Secretary of the Interior in states without approved programs.

39)	“INCREASE IN REGIONAL FLOOD HEIGHT” – A calculated upward rise in the regional flood elevation, equal to or greater than 0.01 foot, based on a comparison of existing conditions and proposed conditions which are directly attributable to development in the floodplain but not attributable to manipulation of mathematical variables such as roughness factors, expansion and contraction coefficients and discharge.

40)	“LAND USE” – Any nonstructural use made of unimproved or improved real estate.  (Also see DEVELOPMENT)

41)	“MANUFACTURED HOME” – A structure transportable in one or more sections, which is build on a permanent chassis and is designed to be used with or without a permanent foundation when connected to required utilities.  The term “manufactured home” includes a mobile home but does not include a “mobile recreational vehicle.”

42)	“MOBILE RECREATIONAL VEHICLE” – A vehicle which is built on a single chassis, 400 square feet or less when measured at the largest horizontal projection, designed to be self-propelled, carried or permanently towable by a licensed, light-duty vehicle, is licensed for highway use if registration is required and is designed primarily not for use as a permanent dwelling, but as temporary living quarters for recreational, camping, travel or seasonal use.  Manufactured homes that are towed or carried onto a parcel of land, but do not remain capable of being towed or carried, including park model homes, do not fall within the definition of “mobile recreational vehicles.”

43)	“MUNICIPALITY” or “MUNICIPAL” – The county, city or village governmental units enacting, administering and enforcing this zoning ordinance.

44)	“NGVD” or “NATIONAL GEODETIC VERTICAL DATUM” – Elevations referenced to mean sea level datum, 1929 adjustment.

45)	“NEW CONSTRUCTION” – For floodplain management purposes, “new construction” means structures for which the start of construction commenced on or after the effective date of floodplain zoning regulations adopted by this community and includes any subsequent improvements to such structures.  For the purpose of determining flood insurance rates, it includes any structures for which the “start of construction” commenced on or after the effective date of an initial FIRM or after December 31, 1974, whichever is later, and includes any subsequent improvements to such structures.

46)	“NONCONFORMING STRUCTURE” – An existing lawful structure or building which is not in conformity with the dimensional or structural requirements of this ordinance for the area of the floodplain which it occupies.  (For example, an existing residential structure in the floodfringe district is a conforming use.  However, if the lowest floor is lower than the flood protection elevation, the structure is nonconforming.)

47)	“NONCONFORMING USE” – An existing lawful use or accessory use of a structure or building which is not in conformity with the provisions of this ordinance for the area of the floodplain which it occupies.  (Such as a residence in the floodway.)

48)	“OBSTRUCTION TO FLOW” – Any development which blocks the conveyance of floodwaters such that this development alone or together with any future development will cause an increase in regional flood height.

49)	“OFFICIAL FLOODPLAIN ZONING MAP” – That map, adopted and made part of this ordinance, as described in s. 1.5(2), which has been approved by the Department and FEMA.

50)	“OPEN SPACE USE” – Those uses having a relatively low flood damage potential and not involving structures.

51)	“ORDINARY HIGHWATER MARK” – The point on the bank or shore up to which the presence and action of surface water is so continuous as to leave a distinctive mark such as by erosion, destruction or prevention of terrestrial vegetation, predominance of aquatic vegetation, or other easily recognized characteristic.

52)	“PERSON” – An individual, or group of individuals, corporation, partnership, association, municipality or state agency.

53)	“PRIVATE SEWAGE SYSTEM” – A sewage treatment and disposal system serving one structure with a septic tank and soil absorption field located on the same parcel as the structure.  It also means an alternative sewage system approved by the Department of Commerce, including a substitute for the septic tank or soil absorption field, a holding tank, a system serving more than one structure or a system located on a different parcel than the structure.

54)	“PUBLIC UTILITIES” – Those utilities using underground or overhead transmission lines such as electric, telephone and telegraph, and distribution and collection systems such as water, sanitary sewer and storm sewer.

55)	“REASONABLY SAFE FROM FLOODING” – Means base flood waters will not inundate the land or damage structures to be removed from the special flood hazard area and that any subsurface waters related to the base flood will not damage existing or proposed buildings.

56)	“REGIONAL FLOOD” – A flood determined to be representative of large floods known to have occurred in Wisconsin.  A regional flood is a flood with a one percent chance of being equaled or exceeded in any given year, and if depicted on the FIRM, the RFE is equivalent to the BFE.

57)	“START OF CONSTRUCTION” – The date the building permit was issued, provided the actual start of construction, repair, reconstruction, rehabilitation, addition, placement, or other improvement was within 180 days of the permit date.  The actual start means either the first placement of permanent construction on a site, such as the pouring of slab or footings, the installation of piles, the construction of columns, or any work beyond initial excavation, or the placement of a manufactured home on a foundation.  Permanent construction does not include land preparation, such as clearing, grading and filling, nor does it include the installation of streets and/or walkways, nor does it include excavation for a basement, footings, piers or foundations or the erection of temporary forms, nor does it include the installation on the property of accessory buildings, such as garages or sheds not occupied as dwelling units or not part of the main structure.  For an alteration, the actual start of construction means the first alteration of any wall, ceiling, floor or other structural part of a building, whether or not that alteration affects the external dimensions of the building.

58)	“STRUCTURE” – Any manmade object with form, shape and utility, either permanently or temporarily attached to, placed upon or set into the ground, stream bed or lake bed, including, not limited to, roofed and walled buildings, gas or liquid storage tanks, bridges, dams and culverts.

59)	“SUBDIVISION” – Has the meaning given in s. 236.02(12), Wis. Stats.

60)	“SUBSTANTIAL DAMAGE” – Damage of an origin sustained by a structure, whereby the cost of restoring the structure to its pre-damaged condition would equal or exceed 50 percent of the equalized assessed value of the structure before the damage occurred.

61)	“UNNECESSARY HARDSHIP” – Where special conditions affecting a particular property, which were not self-created, have made strict conformity with restrictions governing areas, setbacks, frontage, height or density unnecessarily burdensome or unreasonable in light of the purposes of the ordinance.

62)	“VARIANCE” – An authorization by the board of adjustment or appeals for the construction or maintenance of a building or structure in a manner which is inconsistent with dimensional standards (not uses) contained in the floodplain zoning ordinance.

63)	“VIOLATION” – The failure of a structure or other development to be fully compliant with the floodplain zoning ordinance.  A structure or other development without required permits, lowest floor elevation documentation, floodproofing certificates or required floodway encroachment calculations is presumed to be in violation until such time as that documentation is provided.

64)	“WATERSHED” – The entire region contributing runoff or surface water to a watercourse or body of water.

65)	“WATER SURFACE PROFILE” – A graphical representation showing the elevation of the water surface of a watercourse for each position along a reach of river or stream at a certain flood flow.  A water surface profile of the regional flood is used in regulating floodplain areas.

66)	“WELL” – Means an excavation opening in the ground made by digging, boring, drilling, driving or other methods, to obtain groundwater regardless of its intended use.

Introduced this 6th   day of June, 2007

Adopted this ___ day of June, 2007

Village of Silver Lake

___________________________

ROGER JOHNSON, President

Attest:

____________________________                                                   Voted Aye   ____

TERRY FABER, Clerk/Treasurer                                                        Voted Nay   ____

Abstained   ____

Absent      ____