Ordinance No. 450

An Ordinance to Amend Ordinance No. 445,

Entitled Zoning - Section 6.6 & 6.8,

Floodland Conditional Uses

The Village Board of the Village of Silver Lake, Kenosha County, Wisconsin, do ordain as follows:

6.6 – FLOODLAND CONDITIONAL USES

Open space and related uses in the Floodway District may be permitted for the following uses provided that the applicant shall show that such use or improvements will not impede drainage, will not cause ponding, will not obstruct the floodway, will not increase flood flow velocities, will not increase the flood stage, and will not retard the movement of floodwaters.  When permitted, all structures shall be floodproofed and constructed so as not to catch or collect debris nor be damaged by floodwaters.  Fifteen (15) copies of the Certification of Floodproofing shall be made to the Building Inspector and shall consist of a plan or document certified by a registered professional engineer that the floodproofing measures are consistent with the flood velocities, forces, depths, and other factors associated with the 100-year recurrence interval flood, as follows:

- Navigational structures.

- Public water measuring and control facilities.

- Bridges and approaches.

- Marinas.

- Utilities.

- Park and recreational areas, not including structures.

- Parking lots and loading areas accessory to permitted uses in adjacent districts, not including new or used vehicle sales or storage areas.

- Filling as authorized by the Wisconsin Department of Natural Resources to permit the establishment of approved bulkhead lines.

- Other open space uses consistent with the purpose and intent of the district and compatible with uses in adjacent districts, not including structures.

Accessory structures in the Conservancy – Floodplain District provides that all structures, when permitted, shall be floodproofed and constructed so as not to catch or collect debris nor be damaged by floodwaters.  Fifteen (15) copies of the Certification of Floodproofing shall be made to the Building Inspector and shall consist of a plan or document certified by a registered professional engineer that the floodproofing measures are consistent with the flood velocities, forces, depths and other factors associated with the 100-year recurrence interval flood.

Residential structures in the Urban Floodplain Regulatory Area provided that the structure is permitted in the underlying basic use district, and provided that such floodplain fringe areas shall be filled to an elevation at least two (2) feet above the elevation of the 100-year recurrence interval flood.  Such fill shall extend for at least fifteen (15) feet beyond the limits of the structure placed thereon and shall be contiguous to lands outside the floodplain where the depth and duration of floodwaters are sufficient to cause rescue and relief problems.  The finished surface of the lowest floor (including basement) shall be constructed or placed at an elevation that is at least two (2) feet above the elevation of the 100-year recurrence interval flood.  Residential structures placed on fill may be removed from the Urban Floodplain Regulatory Area provided that the fill is contiguous to lands lying outside of the floodlands, and further provided that the property owner, or his agent, has complied with all the requirements for amending the zoning map as set forth in Section 2(7) of this ordinance.

Commercial, industrial and other non-residential structures in the Urban Floodplain Regulatory Area provided that the structure is permitted in the underlying district and provided that the fill requirements for residential structures in the Urban Floodplain Regulatory Area are complied with.  However, when the intent and purpose of this ordinance cannot be fulfilled by filling the floodplain fringe due to existing and committed development, and when the Zoning Board of Appeals has made a finding to this effect, all new structures and all additions to existing structures in the Urban Floodplain Regulatory Area shall be floodproofed to a point two (2) feet above the elevation of the 100-year recurrence interval flood.  Certification of floodproofing shall be made to the Building Inspector and shall consist of a plan or document certified by a registered professional engineer that the floodproofing measures are consistent with the flood velocities, forces, depths and other factors associated with the 100-year recurrence interval flood level for the particular stream reach.  Structures placed on fill may be removed from the Urban Floodplain Regulatory Area provided that the fill is contiguous to lands lying outside of the floodlands, and further provided that the property owner, or his agent, has complied with all the requirements for amending the zoning map as set forth in Section 2(7) of this ordinance.

Mobile homes and mobile home parks provided that the use is permitted in the underlying basic use district; and further provided that the following design and construction standards are complied with;

- All mobile homes shall be placed on a concrete pad constructed on fill or on pilings so that the pad elevation is at least one (1) foot above the elevation of the 100-year recurrence interval flood.  Fill shall be required to extend at least fifteen (15) feet beyond the limits of the concrete pad.

- All mobile homes shall have the lowest floor elevation of the mobile home unit at least two (2) feet above the elevation of the 100-year recurrence interval flood.

- Mobile home units placed on pilings shall be placed on lots that are large enough to permit steps.

- Where pilings are constructed, they shall rest on stable soil, shall not be greater than ten (10) feet apart, and pilings extending more than six (6) feet above the ground level shall be reinforced.

- Mobile home units shall be anchored to resist flotation, collapse or lateral movement.  Anchoring shall consist of over-the-top and frame ties to ground anchors.  Over-the-top ties shall be installed at each of the four (4) corners of the mobile home unit with two (2) additional ties per side at intermediate points.  Only one (1) additional over-the-top tie per side shall be required for mobile home units less than 50 feet in length.  Frame ties shall be required at each of the four (4) corners of the mobile home unit with five (5) additional ties per side at intermediate points.  Only four (4) additional frame ties shall be required for mobile home units less than 50 feet in length.  All components of the anchoring system shall be capable of carrying a force of 4,800 pounds.  Any additions to the mobile home shall be similarly anchored.

- All mobile home parks shall be adequately drained and appropriate access for a hauler shall be provided.

Certification of anchoring, pier construction and pad construction shall be made to the Zoning Administrator and shall consist of a plan or document certified by a registered professional engineer that the anchoring, pier and/or pads meet the minimum requirements set forth herein, and will withstand the flood velocities, forces, depths and other factors associated with the 100-year recurrence interval flood.  Mobile home sites placed on fill may be removed from the Urban Floodplain Fringe Overlay District provided that the fill is contiguous to lands lying outside of the floodlands, and further provided that the property owner, or his agent, has complied with all the requirements for amending the zoning map as set forth in Section 2(7) of this ordinance.

Municipal water supply and sanitary sewerage systems in any Floodland District provided that the system is floodproofed to an elevation at least two (2) feet above the elevation of the 100-year recurrence interval flood, and is designed to eliminate or minimize infiltration of floodwaters into the system.  Certification of floodproofing shall be made to the Building Inspector and shall consist of a plan or document certified by a registered professional engineer that the floodproofing measures are consistent with the flood velocities, forces, depths and other factors associated with the 100-year recurrence interval flood level for the particular stream reach.

Filling to remove lands from the Urban Floodplain Regulatory Area provided that such fill shall be to an elevation at least two (2) feet above the elevation of the 100-year recurrence interval flood and further provided that such lands are contiguous to lands lying outside of the floodlands.  No such Floodplain Fringe Overlay District shall be removed from the Supplementary Floodland Zoning Map until the filling is complete and until the property owner, or his agent, has complied with all the requirements for amending the zoning map as set forth in Section 2(7) of this ordinance.

6.8 – NOTICE TO DNR

The Zoning Board of Appeals shall transmit a copy of each Application for a Conditional Floodland Use to the Wisconsin Department of Natural Resources (DNR) for review and comment.  Final action on the application shall not be taken for thirty (30) days or until the DNR has made its recommendation, whichever comes first.  A copy of all decisions relating to conditional uses in floodland districts shall be transmitted to the DNR within ten (10) days of the effective date of such decision.

This ordinance shall take effect and be in force from and after its passage, approval and publication or posting as provided by law.

Introduced this 18th day of May 2005.

Adopted this 1st day of June 2005.

_________________________________

JEFFREY A. ALBRECHT, President

Attest:

____________________________________

PATRICIA A. LEBER, Clerk-Treasurer					Voted Aye 	  6_

Voted Nay 	  0_

Abstain     	  0_

Absent      	  1_